Matrix Multiplication Program

Program Contents

This program permits a robot linkage to be described

using a series of rotations and translations.  In addition,

it requires CGA compatible graphics or better.

Operations are performed using the multiplication of

Homogeneous Transformation Matrices or HTM's.  Each of the

HTM's may be viewed and modified.  In addition, the partial

products are available for determining intermediate positions

along a robot frame.  End position of the robot hand as

well as its roll, pitch and yaw are calculated.  Graphic views

are then provided to confirm proper data entry.

Eight options were originally presented by the program.

The first of these allows link matrices to be created by simply

entering a desired translation or rotation.  If you are

unsure at the outset just how many rotations or translations

will be needed then enter a slightly larger number.  A "do

nothing" operation consisting of a translation of zero units

or a rotation of zero degrees can then be used to fill the

unwanted positions.

Option 2 allows you alter or view any of the existing

transformations.  This option asks which transform you wish

to see first.  After responding you may scroll forward or

backward through each of the HTM's in the multiplication

sequence by typing "H" or"L."  To modify the matrix you are

looking at type "R."  Once the matrices are corrected,

type "E" to exit this mode.  The robot is replotted and you

are returned to the main menu.  No remnant of the old matrix

exists.

Option 3 sends the HTM's to a parallel printer port.

By contrast, option 4 lets you print the partial products of

all matrices up to any particular location within the robot.

This lets you calculate the x,y,z position of each joint

within the robot.  Option 5 permits the student to obtain an

estimate of the roll, pitch and yaw of the "hand axis" with

respect to the base of the robot.  This part of the program

is readily modified to accommodate other orientation

conventions (IEEE Robotics Tutorial, Fu and Lee, IEEE Press,

1982).

Option 6 displays the final product or hand transform.

Along the right hand side of this matrix are the x, y, and z

coordinates of the hand.  The nine positions to the immediate

right of the position data contain direction cosine

information that describes the angles between the frame of

reference of the base and the frame of reference of the

hand.

Option 7 shows the conventions used by this program to

display the HTM information.The computer plots the linkages you

have created using  coordinate conventions that are listed by

option 7 of the program.  To accommodate robot models of various

sizes, the screen uses approximately 10 units per tick mark.

Although usually unnecessary, you may wish to scale your data up

or down by a factor of ten to accommodate screen size.  It is

strongly recommended that a small wire "axis model" or "triaxis"

be created that can be moved along the robot frame starting from

the base of the robot and moving toward the hand.  This gives a

good understanding of just what transform operations need to be

performed in order to calculate the end position of the hand.  It

also helps illustrate that a number of different possible routes

can be taken to obtain the same end result.  To reduce

confusion, you may wish to adapt a link convention such as

suggested by Paul (Robot Manipulators, Paul, R.P., MIT Press,

1981).

Option 8 graphically shows the calculated trajectory.

Option 9 allows you to save your data to disk.  Eight letter,

DOS compatible names must be used.

Option 10 allows you to recall your data.

Option 11 allows you to exit back to DOS or to the main menu

selection program.

Program Use

To illustrate a typical use of the program  first note that a

rotation about any axis is defined as positive if, as we look down

the positive axis of rotation toward the origin, the rotation is

counter clockwise.  Next, assume that you are a traveller along

the body of the robot or structure.  Out of your nose comes an x

axis, out of the top of your head comes a z axis and with your

arms straight out, your left arm is a positive y axis.

To enter the data for a typical linkage system chose option 1

of the program.  Enter the number 5 as the total expected

number of translations and rotations.  Next, enter the

sequence of activities shown below.

Translate +30 units along your x-axis

Rotate +45 degrees about your z-axis

Translate +25 units along your x-axis

Rotate +90 degrees about your z-axis

Translate +10 units along your x-axis

Note that when you have completed entry, the computer

screen should show you a representation of simple robot like stick

figure representing the journey you have just described.

Option 6 allows the final product of all the HTM's to be

viewed on the screen.  This matrix contains the x,y,z

coordinates of the hand and the orientation of the traveller at

the end of his journey, the "direction cosines" if you are dealing

with robotic arm geometry.  The x, y and z coordinates should agree

closely with what you see in the graphs and will appear

down the right hand side of the matrix.  You can convert the

direction cosines into roll, pitch, and yaw using option 5.

Homogeneous Transformation Matrices can have several

possible formats. Option 7 shows the format used.  The

frame of reference of the hand as given by option 6 of the

program lists not only the location with respect to the base

frame but also the difference in angle between the base frame

and the hand as reflected by the direction cosines.

Practice Problems

1. An observer starts at point A and rotates 25 degrees left,

so that his new reference frame becomes x y .  With an x-axis

extending from his nose, and a y-axis coming out of his left

arm he proceeds 20 units forward to point B.  Turning to his

right he will now be oriented as shown at x y .  Proceeding

down his own x-axis, he arrives at point C with orientation

x y .

a) Break this trip down into a series of rotations and trans-

lations and list these steps.

b) Find the x,y coordinates for the observer at the end of

his travel.

c) Define the roll, pitch and yaw of this individual with

respect to his base frame.

d) What are the various positions where he changed course?

2. Show, by multiplying two matrices, that whenever a translation

precedes a rotation in the matrix list, the two sets of opera-

tions can be substituted into a single matrix;  but, if the

order of operations is reversed, the rotation and translation

information cannot be substituted into a single matrix to

accomplish the multiplication.  (For robot applications this

can pose a substantial time savings during calculation.  Notice

also that row 4 multiplications in the matrices can usually be

eliminated since these always result in the same pattern of

zeroes and ones.  In reality, the use of HTM techniques has been

limited due to speed.  These calculations must often be done in

machine language and are often used not for real-time trajectory

control but for target calculation.)

3.  Build a robot model (e.g. Use heavy wire to build a small model.

Wear safety glasses to prevent eye injury.  Glue graph paper to

cardboard and pass the body of the model through the origin of

the graph.  Tape the unused wire in the base of the 'robot' to

the non-graph side of the cardboard using duct tape.)  Confirm

that you can find the end position of the model arm using matrix

multiplication.  The robot should possess a minimum of four and

a maximum of seven links.  No joint angle should equal zero.  Turn

in your model and on a separate piece of paper turn in your

proposed transformations along with a copy of the hand matrix.

As a possible quiz your instructor may ask you to trade models

with other students.  For full credit please label your model

clearly with base frame and hand axis information, joint

angles, joint type (revolute, twisting rotary, linear-see

Industrial Robots, Groover et al, 1986, McGraw Hill), and

joint lengths in millimeters.  As a self check, do your

partial products accurately describe your chosen pivot

points?

4.  Describe the structure obtained from the following

transformations:

Rotate z-axis 30 degrees

Translate z-axis 40 units

Rotate y-axis -45 degrees

Translate z-axis 20 units

Rotate y-axis -45 degrees

Translate z-axis 10 units

What is the end position?  Draw a picture of the

structure looking down the positive y-axis of the base frame.

Most other options out of this program are self explanatory.

For further information the user should consult a textbook on

robotics.